# 5-row P2 connector map to two S-LINK LDCs

|  |  | Z | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | LD3 | LD2 | Vcc (1) | LD1 | LD0 |
|  | 2 | Gnd (2) | LD5 | Gnd (1) | Gnd | LD4 |
|  | 3 | LD7 | Gnd | (1) | LD6 | Gnd |
|  | 4 | Gnd (2) | LD10 | (1) | LD9 | LD8 |
|  | 5 | LD14 | LD13 | (1) | LD12 | LD11 |
|  | 6 | Gnd (2) | LD17 | (1) | LD16 | LD15 |
|  | 7 | LD20 | LD19 | (1) | Gnd | LD18 |
| LDC | 8 | Gnd (2) | LD23 | (1) | LD22 | LD21 |
| SLOT | 9 | LD25 | Gnd | (1) | LD24 | Gnd |
| A | 10 | Gnd (2) | LD28 | (1) | LD27 | LD26 |
|  | 11 | LCTRL# | LD31 | (1) | LD30 | LD29 |
|  | 12 | Gnd (2) | LWEN# | Gnd (1) | Gnd | UDW0 |
|  | 13 | LCLK | Gnd | Vcc (1) | UDW1 | UTDO# |
|  | 14 | Gnd (2) | LDOWN# | (1) | UXOFF# | URESET# |
|  | 15 | URL1 | LDERR# | (1) | URL0 | Gnd |
|  | 16 | Gnd (2) | URL3 | (1) | URL2 | Gnd |
|  | 17 | LD3 | LD2 | (1) | LD1 | LD0 |
|  | 18 | Gnd (2) | LD5 | (1) | Gnd | LD4 |
|  | 19 | LD7 | Gnd | (1) | LD6 | Gnd |
|  | 20 | Gnd (2) | LD10 | (1) | LD9 | LD8 |
|  | 21 | LD14 | LD13 | (1) | LD12 | LD11 |
|  | 22 | Gnd (2) | LD17 | Gnd (1) | LD16 | LD15 |
|  | 23 | LD20 | LD19 | (1) | Gnd | LD18 |
| LDC | 24 | Gnd (2) | LD23 | (1) | LD22 | LD21 |
| SLOT | 25 | LD25 | Gnd | (1) | LD24 | Gnd |
| B | 26 | Gnd (2) | LD28 | (1) | LD27 | LD26 |
|  | 27 | LCTRL# | LD31 | (1) | LD30 | LD29 |
|  | 28 | Gnd (2) | LWEN# | (1) | Gnd | UDW0 |
|  | 29 | LCLK | Gnd | (1) | UDW1 | UTDO# |
|  | 30 | Gnd (2) | LDOWN# | (1) | UXOFF# | URESET# |
|  | 31 | URL1 | LDERR# | Gnd (1) | URL0 | Gnd (2) |
|  | 32 | Gnd (2) | URL3 | Vcc (1) | URL2 | Vpc (2) |

(1) signals defined by standard VME bus specification. On P2 backplane only available

on front side

(2) signals defined by VME64x (not xP) specification

If signals are free on row B, ground the following pins (in priority order): B16, B5, B26, B8

The same mapping may be used if the P3 backplane is like a P2 backplane.

# 5-row P3 connector map to one S-LINK LDC and one LSC
(uses P2-type backplane)

|  |  | Z | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | LD3 | LD2 | Vcc (1) | LD1 | LD0 |
|  | 2 | Gnd (2) | LD5 | Gnd (1) | Gnd | LD4 |
|  | 3 | LD7 | Gnd | (1) | LD6 | Gnd |
|  | 4 | Gnd (2) | LD10 | (1) | LD9 | LD8 |
|  | 5 | LD14 | LD13 | (1) | LD12 | LD11 |
|  | 6 | Gnd (2) | LD17 | (1) | LD16 | LD15 |
|  | 7 | LD20 | LD19 | (1) | Gnd | LD18 |
| LDC | 8 | Gnd (2) | LD23 | (1) | LD22 | LD21 |
| SLOT | 9 | LD25 | Gnd | (1) | LD24 | Gnd |
| C | 10 | Gnd (2) | LD28 | (1) | LD27 | LD26 |
|  | 11 | LCTRL# | LD31 | (1) | LD30 | LD29 |
|  | 12 | Gnd (2) | LWEN# | Gnd (1) | Gnd | UDW0 |
|  | 13 | LCLK | Gnd | Vcc (1) | UDW1 | UTDO# |
|  | 14 | Gnd (2) | LDOWN# | (1) | UXOFF# | URESET# |
|  | 15 | URL1 | LDERR# | (1) | URL0 | Gnd |
|  | 16 | Gnd (2) | URL3 | (1) | URL2 | Gnd |
|  | 17 | UD3 | UD2 | (1) | UD1 | UD0 |
|  | 18 | Gnd (2) | UD5 | (1) | Gnd | UD4 |
|  | 19 | UD7 | Gnd | (1) | UD6 | Gnd |
|  | 20 | Gnd (2) | UD10 | (1) | UD9 | UD8 |
|  | 21 | UD14 | UD13 | (1) | UD12 | UD11 |
|  | 22 | Gnd (2) | UD17 | Gnd (1) | UD16 | UD15 |
|  | 23 | UD20 | UD19 | (1) | Gnd | UD18 |
| LSC | 24 | Gnd (2) | UD23 | (1) | UD22 | UD21 |
| SLOT | 25 | UD25 | Gnd | (1) | UD24 | Gnd |
| D | 26 | Gnd (2) | UD28 | (1) | UD27 | UD26 |
|  | 27 | UCTRL# | UD31 | (1) | UD30 | UD29 |
|  | 28 | Gnd (2) | UWEN# | (1) | Gnd | UDW0 |
|  | 29 | UCLK | Gnd | (1) | UDW1 | UTEST# |
|  | 30 | Gnd (2) | LDOWN# | (1) | LFF# | URESET# |
|  | 31 | LRL1 | NC (3) | Gnd (1) | LRL0 | Gnd (2) |
|  | 32 | Gnd (2) | LRL3 | Vcc (1) | LRL2 | Vpc (2) |

(1) signals defined by standard VME bus specification. On P2-like backplane only available

on front side

(2) signals defined by VME64x (not xP) specification

(3) NC: do not connect (corresponds to LDERR# line on LDC connector)

If signals are free on row B, ground the following pins (in priority order): B16, B5, B26, B8

Erik van der Bij - 17 January 2000